CSCE 347 Exam Information Sheet

GPIO

Initialization Ritual for GPIO (executed once at beginning)

  - Turn on Port x clock in SYSCTL_RCGCGPIO_R/ SYSCTL_RCGC2_R  Wait two bus cycles (two NOP)

  - For Port F or Port D only: Unlock PF0 (PD7 also needs unlocking)

  - Clear AMSEL to disable analog

  - Clear PCTL to select GPIO

  - Set DIR to 0 for input, 1 for output

  - Clear AFSEL bits to 0 to select regular I/O

  - Set PUR bits to 1 to enable internal pull-up

  - Set DEN bits to 1 to enable data pins

PLL and SysTick

Initialize PLL: Use RCC2 because it provides for more options.

  - Set BYPASS2 (bit 11). At this point the PLL is bypassed and there is no system clock divider.

  - Specify the crystal frequency in the four XTAL bits using the code in Table 10.1. The OSCSRC2 bits are cleared to select the main oscillator as the oscillator clock source.

  - Clear PWRDN2 (bit 13) to activate the PLL.

  - Configure and enable the clock divider using the 7-bit SYSDIV2 field.

  - Wait for the PLL to stabilize by waiting for PLLRIS (bit 6) in the SYSCTL_RIS_R to become high.

  - Connect the PLL by clearing the BYPASS2 bit.

Table 4.7b. Main clock registers for TM4C123

Table 4.7a. XTAL field used in the SYSCTL_RCC_R register of the TM4C123

Initialize SysTick

  - Clear ENABLE to stop counter

  - Update RELOAD value

  - Clear the counter via NVIC_ST_CURRENT_R

  - Set CLK-SRC=1 and specify interrupt action via INTEN in NVIC_ST_CTRL_R

  - Note operation of phase-locked loop (PLL) in determining clock period

SysTick Registers

Interrupt related tables: Table 9.1 – Table 9.6

Table 9.2 The TM4C NVIC registers. Each register is 32 bits wide. Bits not shown are zero.

Table 9.3 Two of the five NVIC interrupt enable registers.

Table 9.1 Some of the interrupt vectors for the TM4C123.

Table 9.4 Some TM4C123 Port A registers.

GPIO_PORTx_ICR_R: set by software for acknowledge

GPIO_PORTx_IRS_R: set by hardware when triggered, cleared by hardware when software sets corresponding GPIO_PORTx_ICR_R bit(s).

Table 9.5 Edge-triggered and level-active interrupt modes (set IME=1 to arm interrupt)

Table 9.6 	SysTick Registers

ADC Registers

Table 10.3. The TM4C123/LM4F120ADC registers. Each register is 32 bits wide.

Set MAXADCSPD to 00 for slow speed operation. The ADC has four sequencers, but we will use only sequencer 3. We set the ADC_SSPRI_R register to 0x3210 to make sequencer 3 the lowest priority. Because we are using just one sequencer, we just need to make sure each sequencer has a unique priority. We set bits 15–12 (EM3) in the ADC_EMUX_R register to specify how the ADC will be triggered. If we specify software start (EM3=0x0), then the software writes an 8 (SS3) to the ADC_PSSI_R to initiate a conversion on sequencer 3. Bit 3 (INR3) in the ADC_RIS_R register will be set when the conversion is complete. We can enable and disable the sequencers using the ADC_ACTSS_R register. There are 11 on the TM4C123/LM4F120. Which channel we sample is configured by writing to the ADC_SSMUX3_R register. The ADC_SSCTL3_R register specifies the mode of the ADC sample. Clear TS0. We set IE0 so that the INR3 bit is set on ADC conversion, and clear it when no flags are needed. We will set IE0 for both interrupt and busy-wait synchronization. When using sequencer 3, there is only one sample, so END0 will always be set, signifying this sample is the end of the sequence. Clear the D0 bit. The ADC_RIS_R register has flags that are set when the conversion is complete; assuming the IE0 bit is set. Do not set bits in the ADC_IM_R register because we do not want interrupts. Write one to ADC_ISC_R to clear the corresponding bit in the ADC_RIS_R register.